# Dual-Mode Power Architecture: 12V DC Battery & Solar-Grid Hybrid Setup

## 1. Direct 12V DC Battery Configuration (Off-Grid)

* **Battery Bank:** Configured to run on standard deep-cycle lead-acid or LiFePO4 storage batteries (e.g., dual 12V 150Ah–200Ah tractor or solar inverter batteries connected in parallel).

* **Direct DC Operation:** Powers the 12V/24V DC PTC air heaters and blowers directly, bypassing inverters to eliminate DC-to-AC conversion losses.

* **Switching:** Managed via heavy-duty 12V DC Solid State Relays (SSRs) connected to the digital thermostat.

* **Autonomy:** Provides continuous, uninterrupted operation for a full 8-to-10-hour commercial drying cycle independently of the local grid.

## 2. Solar-Grid Hybrid Setup (Smart Switching)

* **Primary Solar PV Array:** A dedicated solar array (400W–600W capacity) connected via an MPPT solar charge controller. During peak sunlight, solar power directly drives the heaters and blowers while trickle-charging the battery bank.

* **Secondary Grid Backup:** An automated dual-power transfer switch or smart SMPS (Switched-Mode Power Supply) tied to a standard 230V AC grid supply.

* **Fail-Safe Operation:** If cloud cover reduces battery voltage below a safe threshold (e.g., 11.5V), the system seamlessly switches to grid power, ensuring commercial processing schedules are never compromised by weather fluctuations.